# CALIFORNIA CITRUS NURSERY BOARD

Final Report (year 3 of 4)

12-31-2012

The Future of the Cooperative Registration Program of Nursery Owned Citrus Source Trees

Dr. Georgios Vidalakis & Dr. Fatima Osman

_________________________________________________________________________________

# Objectives

Development, validation, and employment of broad range/universal molecular diagnostic techniques for citrus pathogens of regulatory importance:

- Reverse transcription real-time quantitative polymerase chain reaction (RT-qPCR) protocols for the RNA pathogens:

  - Citrus viroids (exocortis, cachexia, and other abnormalities)

  - Citrus tristeza virus (quick decline, stem pitting, and seedling yellows)

  - Citrus psorosis virus (seed transmission)

  - Citrus leaf blotch virus (dweet mottle virus-seed transmission)

- qPCR protocols for the DNA pathogens:

  - Candidatus Liberibacter sp. (Huanglongbing)

  - Spiroplasma citri (stubborn)

_________________________________________________________________________________

We fine-tuned and used extensively the previously developed nucleic acid extraction and purification procedure from citrus tissue using the semi-automated high throughput systems Geno Grinder 2010 and MagMAXTM Express-96. The high throughput automation system has been enhanced by the addition of a robotic liquid handler Beckman Coulter Biomek 4000 for standardization and optimization of the method. We made significant progress towards the overall goal for a unified high throughput RT-qPCR and qPCR testing, preferably multiplex qPCR-based system for the CDFA Citrus Nursery Stock Pest Cleanliness Program.

- Development of RT-qPCR protocols for RNA citrus pathogens

- Citrus viroids RT-qPCR assay-100% Completed

Seven distinct viroid species representing four genera of the Pospiviroidae family have been identified in citrus. Citrus exocortis viroid (CEVd, genus Pospiviroid), Hop stunt viroid (HSVd, genus Hostuviroid), Citrus bark cracking viroid (CBCVd, genus Cocadviroid) and Citrus bent leaf viroid (CBLVd), Citrus dwarfing viroid (CDVd), Citrus viroid V (CVd-V) and CVd-VI of the genus Apscaviroid cause various citrus diseases and abnormalities. We developed two sets of degenerate primers named “Apsca-Group” and “Non-Apsca-Group” and their respective SYBR Green RT-qPCR protocols capable of detecting all known citrus viroids in two reactions.

As of September 10, 2012 the CDFA approved the testing protocol developed with this project for use in the Citrus Nursery Stock Pest Cleanliness Program (PERMIT NO. QC 1354). A patent application (U.S. 61/556634) has been submitted and communication with a diagnostics company has begun for the commercialization of the method. The peer reviewed publication for the method is currently under preparation.

- Citrus tristeza virus (CTV) RT-qPCR assay-65% Complete

During this past year based on experimental and phylogenetic analysis data as well as practical issues related to quarantine restrictions and requirements we realized that we had to approach the development of RT-qPCR CTV detection assays at two levels.

- Universal CTV detection for endemic California isolates for the immediate development of a method applicable to the CDFA citrus nurseries cleanness program.

- Universal CTV detection of all CTV isolates California endemic and exotic for an assay that can be applied to the registration testing program at a later time as a safety net against the introduction of exotic CTV isolates.

- Universal CTV detection for endemic California isolates

An alignment of California CTV Coat Protein (CP) isolates derived from the CCPP disease bank, the CCTEA collection and 2 complete genome sequences from the GenBank L12175 (16670 K) and M76485 1670 K was produced (Table 1).

# Table1. Summary of CTV sequences used for the CTV-CA-CP alignment.

| # CTV Isolates | # Number of CP sequences |
| --- | --- |
| # CCTEA | # 180 |
| # UCR-CCPP | # 65 |
| # GenBank sequences (L12175 + M76485) | # 2 |
| # Total sequence pile up: | # 247 |

From that alignment, a CTV RT-qPCR assay was generated and was labeled CTV-CA-CP (Fig. 1). This assay was validated for its and sensitivity in detection (Table 1, selected CTV positive controls).

|  | Slope: 1.07 Efficiency: 91.1 |
| --- | --- |
| Fig. 1. Standard Curve & Amplification Efficiency Calculation. Validation-Efficiency of the CTV-CA-CP, RT-qPCR assay using a 10 fold serial dilution standard curve using CTV T521 of the T30 genetic group; the most typical group in California CTV population (see also Table 2. T521 outperforms other tested isolates with the lowest Cq value). Note the high (close to 1) R2 value and the increased Cq values as dilution level increases and concentration of RNA target decreases. |  |

The CTV-CA-CP RT-qPCR assay was tested for its sensitivity in detecting three representative California CTV isolates selected based on CTV genetic groups and phylogenetic analysis (Table 2).

Table 2. Validation-Efficiency and sensitivity of the CTV-CA-CP, RT-qPCR assay using California CTV isolates.

| CTV Isolate: | T528 | SY555 | T521 | H20 |
| --- | --- | --- | --- | --- |
| Genetic group: | T30 | VT & T68 | T30 | NA |
| Cq values: | 22.18 | 20.74 | 20.70 | - |
| Efficiency of qPCR: | 91.10% |  |  |  |

- Universal CTV Global detection of all CTV isolates (California endemic and exotic)

The CCPP Citrus Diagnostic and Research Laboratory at UCR applied for a USDA-APHIS-PPQ 526 permit in January 14, 2012 in order to receive exotic to California CTV isolates to process them with the newly developed high throughput robotic nucleic acid extraction protocol and use them as targets to develop and validate the universal CTV RT-qPCR assay. The permit was issued on September 06, 2012. This delay was one of the primary reasons for the decision to proceed with the immediate development of a CTV RT-qPCR assay to detect the California endemic CTV isolates (see previous section) for the CDFA approval in the registration testing and proceed with Universal CTV detection assay at a later time.

A total of 2174 CTV sequences are available in the GenBank. Of which 898 Accessions are CTV Coat protein (CP) and CP minor (CPm). Sequences were analyzed by separating the CP sequence from the CPm sequences in the GenBank accessions that have the entire genome and trimming sequences that do not align, thus generating 668 CTV CP sequences (Table 3).

# Table 3. Summary of CTV CP sequences used for CTV-Global alignment.

| CTV CP accessions | Number of CP sequences |
| --- | --- |
| GenBank | 668 |
| CCTEA | 180 |
| UCR-CCPP | 65 |
| Total sequence pile up | 913 |

The permit P526P-12-03953 allowed us to import 195 CTV infected samples from Beltsville representing the international genotypes maintained at the Citrus Quarantine Unit, USDA-APHIS. At this point we would like to acknowledge and thank Dr. John Hartung and Cristina Paul for their collaboration. Cristina had prepared all CTV material while the permitting process was underway. The 195 CTV isolates were received and used for the direct validation of the high throughput robotic nucleic acid extraction protocol and the validation of the CTV-CA-CP RT-qPCR assay. CTV CP generated sequences from these CTV isolates have been generated and piled up with the CTV CP alignment to further validate the CTV qPCR based detection assay. Furthermore CTV CP sequences generated by the Illumina analysis for full range of genetic CTV diversity will be generated to develop the Universal CTV qPCR based global detection of all CTV isolates.

- Citrus psorosis virus (CPsV) RT-qPCR assay

DNAstar software was used to align CPsV sequences. This software is capable of aligning a large number of sequences and also checks whether a sequence is reversed in the GenBank or not. It also allows for sorting sequences together. In this case the entire sequences of CPsV in the GenBank were sorted and a CPsV CP sequence alignment was generated (Fig. 2).

The published CPsV qPCR assay (Loconsole, et al., 2010) was also piled up with the generated sequence of the GenBank plus in-house sequence and a number of mismatches were found. Based on the CPsV CP gene sequence pile up, a new CPsV RT-qPCR assay was designed with multiple primers/probes and was labeled CPsV-F.O (Fig. 2).

Fig.2. CPsV CP sequences pile up used for the development of the CPsV-F.O RT-qPCR assay, using two forward primers, one qPCR probe and 2 reverse primers.

Both assays were validated using two formats namely the 2 Step qPCR and the Low Density Array (LDA) format (Table 4), and efficiency has been determined (Fig. 3; Table 4).  A set of conventional RT-PCR was also generated in the CP gene from the sequence pile up of the entire GenBank accession and UCR derived CP samples.

|  | Slope: 1.00 Efficiency: 100.0 |
| --- | --- |
| Fig. 3. Standard Curve & Amplification Efficiency Calculation. Validation-Efficiency of the CPsV-F.O. RT-qPCR assay using a 10 fold serial dilution standard curve using CPsV P205 (see also Table 4. P205 outperforms other tested isolates with lowest Cq value). Note the high (close to 1) R2 value and the increased Cq values as dilution level increases and concentration of RNA target decreases. |  |

Table 4. Validation-Efficiency of the CPsV-F.O. RT-qPCR assay on two platforms, real time RT-qPCR and Low Density Arrays (LDA) using representatives CPsV isolates.

|  |  |  |  |  |  | Efficiency % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CPsV Isolates: | P200A | P201 | P202 | P205 | H20 | RT-qPCR | LDA |
| Cq values: | 25.71 | 23.97 | 24.05 | 22.12 | - | 100 | 92.3 |

Additional samples derived from CDFA and various nurseries have been tested using the in house developed CPsV-F.O. RT-qPCR assay.

d. Citrus leaf blotch virus (CLBV) RT-qPCR assay

There is one report on CLBV RT-qPCR detection (Ruiz-Ruiz et al., 2009) that is designed in the replicase gene. The Ruiz-Ruiz et al., 2009 RT-qPCR assay (CLBV-Ruiz Rep) was found to have mismatches after aligning it to the replicase sequence pile up. Therefore, we have designed three CLBV RT-qPCR assays; two in the CP (CLBV-CP 161 RT-qPCR and CLBV-CP 208 RT-qPCR) and one in the replicase (CLBV-Rep-98) region. Total RNA extracts from the 2011 nursery testing program were used for the validation of the designed RT-qPCR assays.

The three newly designed CLBV RT-qPCR assays were tested for their sensitivity in detecting four CLBV isolates available in the CCPP disease bank (Table 5). Three isolates (DMV 930-932) were intercepted in the CCPP during a variety introduction in the 1960s. The isolate 3069-1 was kindly provided by Dr. Louis Navarro from IVIA-Spain.

Comparing the specificity of all three RT-qPCR assays, CLBV 161 CP was chosen as it produces the lowest Cq values when tested against CLBV isolates as shown in Table 5. Interestingly, CLBV-Ruiz Rep RT-qPCR assay detected only the isolate from Spain (3069-1) with an elevated Cq value (Table 5).

Table. 5. Validation-Efficiency of the CLBV RT-qPCR assays on two platforms, real time RT-qPCR and Low Density Arrays (LDA).

|  | Cq Values of Different CLBV Isolates |  |  |  |  | Efficiency % |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Assay | DMV930 | DMV931 | DMV932 | 3069-1 | H20 | RT-qPCR | LDA |
| CLBV-CP 161 | 22.25 | 21.88 | 22.51 | 23.72 | - | 96 | 96 |
| CLBV-CP 208 | 26.68 | 22.3 | 22.69 | 23.84 | - | 92.3 | 92.3 |
| CLBV-Rep-98 | 21.58 | 20.94 | 24.98 | 22.96 | - | - | - |
| CLBV-Ruiz Rep | - | - | - | 31.96 | - | - | - |

Validation has been performed on CLBV-CP 161 RT-qPCR, as it has produced the best Cq value (Fig. 4).

|  | Slope: 1.03 Efficiency: 96.0 |
| --- | --- |
| Fig. 4. Standard Curve & Amplification Efficiency Calculation. Validation-Efficiency of the CLBV-CP 161 RT-qPCR LDA assay using a 10 fold serial dilution standard curve using DMV931 (see also Table 2. DMV931 outperforms other tested isolates with lowest Cq value). Note the high (close to 1) R2 value and the increased Cq values as dilution level increases and concentration of RNA target decreases. |  |

Additional samples derived from CDFA and various nurseries have been tested using the in house developed CLBV CP 161 RT-qPCR assay.

2. DEVELOPMENT OF qPCR PROTOCOLS FOR DNA CITRUS PATHOGENS

- qPCR diagnostic techniques for the two targeted DNA citrus pathogens, i.e. C. Liberibacter sp. and S. citri have been developed already (Kim et al., 2009; Manjunath et al., 2008 and Li et al., 2006, Yokomi et at., 2008). The missing components for their application into the nursery registration program is the incorporation of the newly developed nucleic acid extraction with the automated system MagMax 96, the validation of the techniques in our laboratory, and eventually the multiplexing for those two DNA pathogens in order to reduce the cost of detection in the program.

- Our objective has been the development of new or adjustment of the existing detection assays only if we identify problems with the detection spectrum of the method i.e. ability to detect various pathogenic isolates and species (i.e. C. Liberibacter asiaticus, africanus, and americanus) or if the combination into a multiplex reaction is not possible.

- Candidatus Liberibacter sp. qPCR assay

Citrus Huanglongbing (HLB) is one of the most serious diseases of citrus. Different forms of the disease are caused by different Candidatus Liberibacter species, Candidatus Liberibacter asiaticus (Las), Ca. L. africanus (Laf) and Ca. L. americanus (Lam).

- Three separate sequence pile ups were generated for each C. Liberibacter species. Sequences were conserved.

- A general sequence pile up of the three species (Las, Laf, and Lam) for the 16 S rRNA sequences was created (Fig. 5). The sequence was much conserved and a HLB qPCR assay was designed.

Fig. 5. Alignment of Candidatus Liberibacter spp. (Asiaticus, Americanus and Africanus)

Three universal qPCR assays (Can. L, Can. L2 and HLB new) were developed on the 16 S rRNA sequence pile up of the three C. liberibacter species (Las, Laf, and Lam). Several samples with known low Cq values were chosen to validate the three C. liberibacter qPCR assays (Can. L, Can. L2 designed by F.O. and the HLB new qPCR assay which has been modified from Li et al., 2006 qPCR assay. The HLB new qPCR assay has been validated and was shown to be more sensitive in detecting the three C. liberibacter species. This universal HLB new qPCR assay was tested against the three Las, Laf and Lam species kindly provided from Florida, South Africa, and Brazil (Fig. 6 and Table 6).

|  | Slope: 0.99 Efficiency: 101.4 |
| --- | --- |
| Fig. 6. Standard Curve & Amplification Efficiency Calculation. Validation-Efficiency of the C. Liberibacter HLB new qPCR assay using a 10 fold serial dilution standard curve using a Can L positive standard. Note the high (close to 1) R2 value and the increased Cq values as dilution level increases and concentration of DNA target decreases. |  |

This universal C. Liberibacter HLB new qPCR assay was capable of detecting all three C. Liberibacter species (Lam, Las and Laf) and was validated against many C. liberibacter samples representing the three spp. (Asiaticus, Americanus and Africanus). This universal C.L. HLB new was capable of successful in detecting all samples representing the three spp. Most importantly this HLB new qPCR assay was able to detect HLB-CA isolate that was found in the backyard of a residence March/April of 2012 in Hacienda Heights of LA County, CA (as shown in Table 6).

Collaborators kindly supplied C. Liberibacter nucleic acids used in the validation experiments. Samples have been sent by: Svetlana Folimonova-Florida, Gerhard Pietersen-South Africa, Nelson Wulff-Brazil and Cristina Paul-Beltsville. We would like also to acknowledge Dr. MaryLou Polek and Dr. Cynthia LeVesque of the CRB for providing the USDA-APHIS-PPQ permit and laboratory facilities for the importation of Las infected lyophilized tissue from Florida in order to test the newly developed high throughput robotic nucleic acid extraction protocol approved for use in the registration testing program.

Table 6. Testing Can L HLB new qPCR assay against Can. Liberibacter samples derived from Florida and Beltsville Maryland (asiaticus), South Africa (africanus) and Brazil (americanus).

Multiplex qPCR assay has been designed to detect C. liberibacter (all three C. Liberibacter species, Lam, Las and Laf) and S. citri. Currently we are evaluating its potential for detecting these two DNA pathogens in mixed infected situation.

Spiroplasma citri qPCR assay

All S. citri GenBank accessions were sorted by the DNAstar software and several contigs containing all possible genome sequences were generated. These contig sequences pertaining to different locations within the S. citri genome were piled up.

Six S. citri qPCR assays were designed in different regions of the S. citri genome, namely S. citri contig 29, contig 26, contig 13, contig 4, Spiralin, and P58. Additionally, the qPCR assay designed by Yokomi et al., 2008 was altered by designing an internal qPCR probe between the forward and the reverse primers and generated two additional qPCR assays (S. citri Yokomi C2 and C4). The S. citri-contig 29 was shown to produce the best Cq results as shown in Table 7.

Table 7. Validation-Efficiency of the S. citri qPCR assays using representatives S. citri isolates.

|  | S. citri Isolates and Cq Values |  |  |  | Validation |
| --- | --- | --- | --- | --- | --- |
| S. citri qPCR Assay | C189 | S600 | S616 | H2O |  |
| Contig 29 | 25.61 | 29.56 | 23.94 | - | 98.6% |
| Contig 26 | 32.92 | 39.26 | 33.88 | - |  |
| Contig 13-Spiralin | 27.59 | 32.9 | 26.96 | - | 105.9% |
| Contig 4 | - | - | 39.17 | - |  |
| Spiralin | 30.85 | 35.46 | 30.38 | - |  |
| P58 | 36.89 | - | 34.47 | - |  |
| Yokomi C2 | 28.6 | - | 26.87 | - |  |
| Yokomi C4 | 31.9 | 31.6 | 28.48 | - |  |

S. citri-Contig 29 qPCR assays was validated with an efficiency of 98.6 % using

10 fold serial dilutions of a positive control (Fig. 7; Table 7).

|  | Slope: 1.01 Efficiency: 98.6 |
| --- | --- |
| Fig. 7. Standard Curve & Amplification Efficiency Calculation. Validation-Efficiency of the of the Spiroplasma citri Contig 29 qPCR assay using a 10 fold serial dilution standard curve using S. citri S616 (see also Table 4; Contig 29-S616 outperforms other tests generating the lowest Cq values). Note the high (close to 1) R2 value and the increased Cq values as dilution level increases and concentration of DNA target decreases. |  |